RFID Development Documentation

V2.5

Table of contents

1.Common method usage process	3

1.1 initialization	3

1.2 Inventory	3

1.3 read tag	4

1.4 write tag	4

1.5 get the power of output	4

1.6 set the power of output	4

2. Interface’s information	4

The callback of 6C tag	4

note：there are some parameters do not work,they have a default value,it is necessary to focus on them in this version。	5

onInventoryTag	5

onInventoryTagEnd	5

3. RFID api	7

3.1 init【initialization】	7

3.2 release【release resources】	7

3.3 disConnect【disconnect rfid module】	8

3.4 isConnected【RFID connection status 】	8

3.5 registerCallback【register CallBack】	8

3.6  startInventory【User define session and target inventory】	9

3.7 stopInventory【stop tag inventory】	9

3.8 readTag【read tag】	9

3.9 readDataByTid【Tag data is read via TID】	10

3.10 inventorySingle【single tag query】	10

3.11 writeTag【Write Tag】	11

3.12 writeTagByTid【Write data to each storage area through tid】	11

3.13 writeEpc【write epc number】	12

3.14 killTag【Kill Tag】	13

3.15 KillbyTID【The tag is destroyed by TID】	13

3.16 lockTag【Lock Tag】	13

3.17 LockbyTID【The tag is locked using the TID】	14

3.18 setQueryMode【config query model】	15

3.19 getQueryMode【get query mode】	15

3.20 getFirmwareVersion【Get Reader Firmware Version.】	16

3.21 setOutputPower【Set output power】	16

3.22 getOutputPower【Query output power】	16

3.23 setFrequencyRegion【Set frequency region】	17

3.24 getFrequencyRegion【Query frequency region.】	17

3.25 addMask【add mask】	18

3.26 clearMask【clear mask】	18

3.27 unregisterCallback【Unregister callback】	18

3.28 getModuleFirmware【Get module firmware information】	19

3.29 setScanInterval【Set the inventory interval】	19

3.30 getScanInterval【Read Inventory Interval】	19

3.31 startRead【start inventory】	20

3.32 scanRfid【Take inventory over a period of time】	20

3.33 setInventoryParameter【configure inventory parameters】	20

3.34 getDeviceId【Gets the serial number】	21

3.35 setCustomRegion【Set custom bands】	21

3.36 getCustomRegion【Get a custom band】	21

3.37 setProfile【Set the link configuration information】	22

3.38 setRange【set the distance】	23

3.39 getRange【get the distance】	23

3.40 GetReaderType【get rfid mode】	23

3.41 getReaderTemperature【Query internal temperature】	24

3.42 iso180006BInventory【Inventory 18000-6B Tag】	24

3.43 iso180006BLockTag【Lock 18000-6B Tag】	24

3.44 iso180006BQueryLockTag【Query 18000-6B Tag】	24

3.45 iso180006BReadTag【Read 18000-6B Tag】	25

3.46 iso180006BWriteTag【Write 18000-6B Tag】	25

Return value error code table	27

- 1.Common method usage process

  - initialization

- private void initRfid() { - USDKManager.getInstance().init(BaseApplication.getContext(),new USDKManager.InitListener() { - @Override - public void onStatus(USDKManager.STATUS status) { - if ( status == USDKManager.STATUS.SUCCESS) { - Log.d(TAG, "initRfid() success."); - mRfidManager = USDKManager.getInstance().getRfidManager(); - }else { - Log.d(TAG, "initRfid fail."); - } - } - }); - }

  - Inventory

- After initializing RfidManager’s instance,you can make an inventory of cargo by using startInventory/startRead,and you can get the result from the callback method of OnInventoryTag,which is defined in the interface of IRfiCallbcak.

- steps：

- Start working：

- Rfidmanager.startInventory(byte session);

- Implements the method onInventoryTag of the interface .

  - read tag

- You can get the tag’s information by using the readTag method of RfidManager.

- steps：

- Read tag：rfidmanager.readTag(epc,btMemBank,btWordAdd,btWordCnt,btAryPassWord)，the method will return what you want to get from the target tag.

  - write tag

- You can write the tag by using the writeTag method of RfidManger。

- Steps：

- Write tag：Rfidmanager.writeTag(epc,btAryPassWord,btMemBank，btWordAdd，btWordCnt，btAryData),the method will return the result of the operation.

- 1.5 get the power of output

- You can get current value of output by using

- RfidManager.getOutputPower,using the getOutputPower method to send command.

- Send the command of getting the power of output

- rfidmanager.getOutputPower()

- 1.6 set the power of output

- You can get current value of output by using the setOutputPower method ,using the getOutputPower method to send command.

- Set output power：rfidmanager.setOutputPower()

- Interface’s information

### The callback of 6C tag

If you need to take inventory, you need to register the callback before inventory.

public interface IRfidCallback {

void onInventoryTag(String EPC,String TID,String RSSI);

void onInventoryTagEnd();

}

- The details about methods of the callback interface。

- note：there are some parameters do not work,they have a default value,it is necessary to focus on them in this version。

- onInventoryTag

| define | void onInventoryTag( String epc_data, String data, String rssi); |  |  |
| --- | --- | --- | --- |
| description | Get the result of making an inventory of cargo |  |  |
| parameter | name | type | note |
|  | epc | String | epc |
|  | data | String | If you choose mixed query mode, that data is exactly what you want.(tid，user) |
|  | rssi | String | rssi |

- onInventoryTagEnd

| define | void onInventoryTagEnd(); |  |  |
| --- | --- | --- | --- |
| description | When you call the method to stop inventory, the program executes this callback, which is the end. |  |  |
| parameters | name | type | note |

- The callback of 6B tag:

- ITag6BCallback {

- public void onRead6BTag( String strData) ;

- public void onWrite6BTag( byte nWriteLen) ;

- public void onLock6BTag(byte nStatus) ;

- public void onLockQuery6BTag( byte nStatus) ;

- public void onExeCMDStatus(byte cmd, byte status);

- public void onInventory6BTag(  String strUID );

- }

      - onRead6BTag

| - define | - void onRead6BTag( String strData); |  |  |
| --- | --- | --- | --- |
| - description | - The result callback of reading 6b-tag |  |  |
| - parameters | - name | - type | - note |
|  | - strData | - String |  |

      - onWrite6BTag

| - define | - void onWrite6BTag( byte nWriteLen); |  |  |
| --- | --- | --- | --- |
| - description | - The result callback of writing 6b-tag |  |  |
| - parameters | - name | - type | - note |
|  | - nWriteLen | - byte |  |

      - onLock6BTag

| - define | - void onLock6BTag( byte nStatus); |  |  |
| --- | --- | --- | --- |
| - description | - The result callback of locking 6b-tag |  |  |
| - parameters | - name | - type | - note |
|  | - nStatus | - byte | - 0：success 1：fail |

      - onLockQuery6BTag

| - define | - void onLockQuery6BTag( byte nStatus); |  |  |
| --- | --- | --- | --- |
| - description | - The result callback of querying 6b-tag |  |  |
| - parameters | - name | - type | - note |
|  | - nStatus | - byte | - 0：locked；1：unlocked |

      - onInventory6BTag

| - define | - void onInventory6BTag( String strUID ); |  |  |
| --- | --- | --- | --- |
| - description | - The result callback of inventorying 6b-tag |  |  |
| - parameters | - name | - type | - note |
|  | - strUID | - String |  |

- RFID api

### 3.1 init【initialization】

| define | public void init(); |  |  |
| --- | --- | --- | --- |
| description | Initialize RFIDManager’s intance。 |  |  |
| parameters | name | type | note |
| return |  |  |  |
| Reference Code | None |  |  |

### 3.2 release【release resources】

| define | public void release(); |  |  |
| --- | --- | --- | --- |
| description | release resources |  |  |
| parameters | name | type | note |
| return |  |  |  |
| Reference Code |  |  |  |

### 3.3 disConnect【disconnect rfid module】

| define | public void disConnect(); |  |  |
| --- | --- | --- | --- |
| description | disconnect rfid module。 |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | None |  |  |

### 3.4 isConnected【RFID connection status 】

| define | public boolean isConnected(); |  |  |
| --- | --- | --- | --- |
| description | Determine the connection status of the module |  |  |
| parameters | name | type | note |
| return (boolean) | success：true ；fail：false ； |  |  |
| Reference Code |  |  |  |

- 3.5 registerCallback【register CallBack】

| define | public void registerCallback(com.ubx.usdk.rfid.aidl.IRfidCallback cb); |  |  |
| --- | --- | --- | --- |
| description | Unregister CallBack。 |  |  |
| parameters | name | type | note |
|  | cb | com.ubx.usdk.rfid.aidl.IRfidCallback | Callback |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.6  startInventory【User define session and target inventory】

| define | public int startInventory( byte btSession); |  |  |
| --- | --- | --- | --- |
| description | User define session and target inventory |  |  |
| parameters | name | type | note |
|  | btSession | byte | Desired session ID. Values(0;1;2;3) |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.7 stopInventory【stop tag inventory】

| define | public int stopInventory(); |  |  |
| --- | --- | --- | --- |
| description | stop tag inventory |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.8 readTag【read tag】

| define | public int readTag( String EPC, byte btMemBank, byte btWordAdd, byte btWordCnt, byte[] btAryPassWord); |  |  |
| --- | --- | --- | --- |
| description | Read Tag. Attention: If two tags have the same EPC, but different read data, then these two tags are considered different tags. |  |  |
| parameters | name | type | note |
|  | epc | String | EPC value |
|  | btMemBank | byte | Tag memory bank(0x00:RESERVED, 0x01:EPC, 0x02:TID, 0x03:USER) |
|  | btWordAdd | byte | Read start address,please see the tag's spec for more information. |
|  | btWordCnt | byte | Read data length,Data length in WORD(16bits) unit. Please see the tag's spec for more information. |
|  | btAryPassWord | byte[] | Access password,4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.9 readDataByTid【Tag data is read via TID】

| define | public String readDataByTid(String TIDStr, byte Mem, byte WordPtr, byte Num, String Password) |  |  |
| --- | --- | --- | --- |
| description | Tag data is read via TID |  |  |
| parameters | name | Type | note |
|  | TIDStr | String | tid |
|  | Mem | byte | The region that needs to be read |
|  | WordPtr | byte | The start address of the data to be read |
|  | Num | byte | Length of data |
|  | Password | String | Access password |
| return (String) | - Null:fail |  |  |
| Reference Code |  |  |  |

### 3.10 inventorySingle【single tag query】

| define | public String inventorySingle( ) |  |  |
| --- | --- | --- | --- |
| description | Only one label is read per call. |  |  |
| parameters | name | type | note |
| return | The onInventoryTag() callback returns the corresponding result. |  |  |
| Reference Code | 。 |  |  |

### 3.11 writeTag【Write Tag】

| define | public int writeTag(String epc byte[] btAryPassWord, byte btMemBank, byte btWordAdd, byte btWordCnt, byte[] btAryData); |  |  |
| --- | --- | --- | --- |
| description | Write Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc |
|  | btAryPassWord | byte[] | Access password, 4 bytes. |
|  | btMemBank | byte | Tag memory bank(0x00:RESERVED, 0x01:EPC, 0x02:TID, 0x03:USER) |
|  | btWordAdd | byte | Write start address,WORD(16 bits). When write EPC area, notice that EPC starts from address 02, the first two 2 words are for PC+CRC. |
|  | btWordCnt | byte | WORD(16 bits), please see the tag's spec for more information. |
|  | btAryData | byte[] | Write data, btWordCnt*2 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.12 writeTagByTid【Write data to each storage area through tid】

| define3 | public int writeTagByTid(String TIDStr,byte Mem, byte WordPtr,byte Password[],String wdata); |  |  |
| --- | --- | --- | --- |
| description | Write data to each memory area. |  |  |
| parameters | name | type | note |
|  | TIDStr | String | The tid number of the label, a hexadecimal string, the length must be an integer multiple of 4 |
|  | Mem | byte | - memory area to be written, - 0- Password area, the first 3 words are the destruction password, the last 3 words are the access password - 1-EPC area - 2-TID area - User area |
|  | WordPtr | int | Write start word address |
|  | Password | String | The access password of the label, in hexadecimal string format。 |
|  | WData | String | The data to be written, a hexadecimal string, the length must be an integer multiple of 4 |
| return (int) | Success:0,fail:-1 |  |  |
| Reference Code |  |  |  |

### 3.13 writeEpc【write epc number】

| define1 | public int WriteEPC(byte epclen,byte epc[],byte Password[]); |  |  |
| --- | --- | --- | --- |
| description | Randomly rewrite the EPC number of a label by broadcasting |  |  |
| parameters | name | type | note |
|  | epclen | byte | length of epc |
|  | epc | byte[] | Input, the EPC number of the label, the size is epclen*2 |
|  | Password | byte[] | Input, the access code for the tag, 4 bytes |
| return (int) | successreturn0。 |  |  |
| Reference Code |  |  |  |

- 3.14 killTag【Kill Tag】

| define | public int killTag(String epc,byte[] btAryPassWord); |  |  |
| --- | --- | --- | --- |
| description | Kill Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc value |
|  | btAryPassWord | byte[] | Kill password,4 bytes |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.15 KillbyTID【The tag is destroyed by TID】

| define | public int KillbyTID(byte tidlen, byte[] tid, byte[] Password); |  |  |
| --- | --- | --- | --- |
| description | The tag is destroyed by TID。 |  |  |
| parameters | name | type | note |
|  | tidlen | byte | TID length |
|  | tid | byte[] | Tid value |
|  | Password | byte[] | Kill password,4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.16 lockTag【Lock Tag】

| define | public int lockTag( String epc, byte[] btAryPassWord, byte btMemBank, byte btLockType); |  |  |
| --- | --- | --- | --- |
| description | Lock Tag |  |  |
| parameters | name | type | note |
|  | epc | String | epc value |
|  | btAryPassWord | byte[] | Access password, 4 bytes. |
|  | btMemBank | byte | Tag memory bank(0x01:User Memory, 0x02:TID Memory, 0x03:EPC Memory, 0x04:Access Password, 0x05:Kill Password) |
|  | btLockType | byte | Lock operation type(0x00:Open, 0x01:Lock, 0x02:Permanent open, 0x03:Permanent lock) |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.17 LockbyTID【The tag is locked using the TID】

| define | public int lockTag(byte tidlen, byte[] tid,byte btMemBank, byte btLockType,byte[] btAryPassWord); |  |  |
| --- | --- | --- | --- |
| description | The tag is locked using the TID。 |  |  |
| parameters | name | type | note |
|  | tidlen | byte | tidlength |
|  | tid | byte[] | Tid |
|  | btMemBank | byte | Label storage area (0x01: User memory, 0x03: TID Memory, 0x03: EPC Memory, 0x04: Access password, 0x05: Kill password) |
|  | btLockType | byte | Lock operation type (0x00: Open, 0x01: locked, 0x03: Permanently open, 0x03: Permanently locked) |
|  | btAryPassWord | byte[] | Access password, 4 bytes. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.18 setQueryMode【config query model】

| define | public void setQueryMode(int mode) |  |  |
| --- | --- | --- | --- |
| description | Config query model |  |  |
| parameters | name | type | note |
|  | Mode | Int、 | 0：epc 1：epc+tid 2：epc+user 3：epc+fasttid |
| return (void) |  |  |  |
| Reference Code | none |  |  |

- 3.19 getQueryMode【get query mode】

| define | public int getQueryMode() |  |  |
| --- | --- | --- | --- |
| description | Get the query mode |  |  |
| parameters | name | type | note |
| return (int) | 0：epc - 1：epc+tid - 2：epc+user - 3：epc+fasttid |  |  |
| Reference Code | none |  |  |

### 3.20 getFirmwareVersion【Get Reader Firmware Version.】

| define | public String getFirmwareVersion(); |  |  |
| --- | --- | --- | --- |
| description | Get Reader Firmware Version. |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.21 setOutputPower【Set output power】

| define | public int setOutputPower(byte btOutputPower); |  |  |
| --- | --- | --- | --- |
| description | This command consumes more than 100mS. If you want you change the output power frequently, please use Cmd_set_temporary_output_power command, which doesn't reduce the life of the internal flash memory. |  |  |
| parameters | name | type | note |
|  | btOutputPower | byte | RF output power: range: 0-33 |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.22 getOutputPower【Query output power】

| define | public int getOutputPower(); |  |  |
| --- | --- | --- | --- |
| description | Query output power。 |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.23 setFrequencyRegion【Set frequency region】

| define | public int setFrequencyRegion( byte btRegion, byte btStartRegion, byte btEndRegion); |  |  |
| --- | --- | --- | --- |
| description | Set frequency region(system default frequencies). |  |  |
| parameters | name | type | note |
|  | btRegion | byte | Spectrum regulation(0x04:EU, 0x06:Ukraine, 0x08:Chinese 1, 0x09:EU3). |
|  | btStartRegion | byte | Start frequency of the spectrum. |
|  | btEndRegion | byte | End frequency of the spectrum,Setup the range of the RF output spectrum. The rules are: 1,Start frequency and end frequency should be in the range of the specified regulation. 2,Start frequency should be equal or lower than end frequency. 3, End frequency equals start frequency means use single frequency point. |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

### 3.24 getFrequencyRegion【Query frequency region.】

| define | public RfidDate getFrequencyRegion(); |  |  |
| --- | --- | --- | --- |
| description | Query frequency region. |  |  |
| parameters | name | type | note |
| return (RfidDate ) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.25 addMask【add mask】

| define | public void addMask(int mem,int startAddress,int len,String data) |  |  |
| --- | --- | --- | --- |
| description |  |  |  |
| parameters | name | Type | note |
|  | mem | int | Filter area |
|  | startAddress | int | Start address |
|  | len | int | Length of data |
|  | data | String | Filter data |
| return (void) |  |  |  |
| Reference Code |  |  |  |

- 3.26 clearMask【clear mask】

| define | public void clearMask(); |  |  |
| --- | --- | --- | --- |
| description | Clear so the mask |  |  |
| parameters | name | Type | note |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.27 unregisterCallback【Unregister callback】

| define | public void unregisterCallback(com.ubx.usdk.rfid.aidl.IRfidCallback cb); |  |  |
| --- | --- | --- | --- |
| description | Unregister callback |  |  |
| parameters | name | type | note |
|  | cb | com.ubx.usdk.rfid.aidl.IRfidCallback | Callback |
| return (void) |  |  |  |
| Reference Code |  |  |  |

### 3.28 getModuleFirmware【Get module firmware information】

| define | public String getModuleFirmware(); |  |  |
| --- | --- | --- | --- |
| description | Get module firmware information |  |  |
| parameters | name | type | note |
| return (String) | module firmware information |  |  |
| Reference Code |  |  |  |

### 3.29 setScanInterval【Set the inventory interval】

| define | public int setScanInterval( int interval) |  |  |
| --- | --- | --- | --- |
| description | Set the query parameters to be used when inventory is enabled |  |  |
| parameters | name | type | note |
|  | interval | int | Inventory interval |
| return (void) | 0:success -1:fail |  |  |
| Reference Code |  |  |  |

### 3.30 getScanInterval【Read Inventory Interval】

| define | public int getScanInterval() |  |  |
| --- | --- | --- | --- |
| description | Read the query used when inventory is enabled parameters |  |  |
| parameters | name | type | note |
| return (int) | 0:success -1:fail |  |  |
| Reference Code |  |  |  |

### 3.31 startRead【start inventory】

| define | public int startRead() |  |  |
| --- | --- | --- | --- |
| description | You can configure your own inventory parameters before calling the method |  |  |
| parameters | name | type | note |
| return (int) | - 0:success - -1:fail |  |  |
| Reference Code | none |  |  |

### 3.32 scanRfid【Take inventory over a period of time】

| define | public void scanRfid() |  |  |
| --- | --- | --- | --- |
| description | Take inventory over a period of time |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | none |  |  |

### 3.33 setInventoryParameter【configure inventory parameters】

| define | public void setInventoryParameter(Rfidparameter parameter) |  |  |
| --- | --- | --- | --- |
| description | configure inventory parameters |  |  |
| parameters | name | type | note |
| return (void) |  |  |  |
| Reference Code | none |  |  |

### 3.34 getDeviceId【Gets the serial number】

| define | public String getDeviceId() |  |  |
| --- | --- | --- | --- |
| description | Gets the serial number |  |  |
| parameters | name | Type | note |
| return (String ) | - the serial number |  |  |
| Reference Code | none |  |  |

### 3.35 setCustomRegion【Set custom bands】

| define | public int setCustomRegion(byte flags, int FreSpace, int FreNum, int StartFre); |  |  |
| --- | --- | --- | --- |
| description | Set custom bands |  |  |
| parameters | name | Type | note |
|  | flags | byte | Whether to save the setting information ID when powered 0：save 1：clear |
|  | int | FreSpace | Band spacing |
|  | int | FreNum | Number of frequency points |
|  | int | StartFre | Starting band |
| return (int) | - 0:success |  |  |
| Reference Code |  |  |  |

### 3.36 getCustomRegion【Get a custom band】

| define | public CustomRegionBean getCustomRegion(); |  |  |
| --- | --- | --- | --- |
| description | Get a custom band information |  |  |
| parameters | name | Type | note |
| return (CustomRegionBean ) | - Null:fail - public class CustomRegionBean { - public int [] band=new int[1];// - public int [] FreSpace=new int[1];// - public int [] FreNum=new int[1];// - public int [] StartFre=new int[1];//d - public CustomRegionBean(){} - } |  |  |
| Reference Code |  |  |  |

### 3.37 setProfile【Set the link configuration information】

| define | public int setProfile(byte param); |  |  |
| --- | --- | --- | --- |
| description | Set the link configuration information |  |  |
| parameters | name | Type | note |
|  | param | byte | Generally, the optional parameters of this parameter need to be provided according to the device information. If the type of reader is（0x21,0x23,0x28,0x36,0x37） parm： 0:40K, FM0,25us 1:250K,M4, 25us 2:300K,M4, 25us 3:400K,FM0,6.25us Other 11:640K,FM0,7.5u 1:640K, M2,7.5us 15:640K, M4,7.5us 12:320K, M2, 15us 3:320K, M2, 20us 5:320K, M4, 20us 7:250K, M4, 20us 13:160K, M8, 20us |
| Void |  |  |  |
| Reference Code |  |  |  |

### 3.38 setRange【set the distance】

| define | public int setRange(int range); |  |  |
| --- | --- | --- | --- |
| description | Set the distance at which the label can be identified |  |  |
| parameters | name | Type | note |
|  | Int | range | 0~100 |
| Int | - 0:success; |  |  |
| Reference Code |  |  |  |

### 3.39 getRange【get the distance】

| define | public int getRange(); |  |  |
| --- | --- | --- | --- |
| description | get the distance at which the label can be identified |  |  |
| parameters | name | Type | note |
| Int | - -1:fail; - Other:the distance |  |  |
| Reference Code |  |  |  |

- 3.40 GetReaderType【get rfid mode】

| define | public int GetReaderType() |  |  |
| --- | --- | --- | --- |
| description | Get rfid mode |  |  |
| parameters | name | type | note |
| return (int) | - 80：the mode of short distance - others：the mode of long distance mode |  |  |
| Reference Code | none |  |  |

- 3.41 getReaderTemperature【Query internal temperature】

| define | public RfidDate getReaderTemperature(); |  |  |
| --- | --- | --- | --- |
| description | Query internal temperature。 |  |  |
| parameters | name | type | note |
| return (int) |  |  |  |
| Reference Code |  |  |  |

- 3.42 iso180006BInventory【Inventory 18000-6B Tag】

| define | public int iso180006BInventory(); |  |  |
| --- | --- | --- | --- |
| description | Inventory 18000-6B Tag |  |  |
| parameters | name | type | note |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.43 iso180006BLockTag【Lock 18000-6B Tag】

| define | public int iso180006BLockTag(byte[] btAryUID,byte btWordAdd); |  |  |
| --- | --- | --- | --- |
| description | Lock 18000-6B Tag。 |  |  |
| parameters | name | type | note |
|  | btAryUID | byte[] | Operated Tag's UID, 8 bytes |
|  | btWordAdd | byte | Locked address |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.44 iso180006BQueryLockTag【Query 18000-6B Tag】

| define | public int iso180006BQueryLockTag( byte[] btAryUID, byte btWordAdd); |  |  |
| --- | --- | --- | --- |
| description | Query locked 18000-6B Tag。 |  |  |
| parameters | name | type | note |
|  | btAryUID | byte[] | Operated Tag's UID, 8 bytes |
|  | btWordAdd | byte | Locked address |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.45 iso180006BReadTag【Read 18000-6B Tag】

| define | public int iso180006BReadTag( byte[] btAryUID, byte btWordAdd, byte btWordCnt); |  |  |
| --- | --- | --- | --- |
| description | Read 18000-6B Tag。 |  |  |
| parameters | name | type | note |
|  | btAryUID | byte[] | Operated Tag's UID, 8 bytes |
|  | btWordAdd | byte | Read data first address |
|  | btWordCnt | byte | Read data length |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

- 3.46 iso180006BWriteTag【Write 18000-6B Tag】

| define | public int iso180006BWriteTag(String epc, byte[] btAryUID, byte btWordAdd, byte btWordCnt, byte[] btAryBuffer); |  |  |
| --- | --- | --- | --- |
| description | Write 18000-6B Tag。 |  |  |
| parameters | name | type | note |
|  | btAryUID | byte[] | Operated Tag's UID, 8 bytes |
|  | btWordAdd | byte | Write data first address |
|  | btWordCnt | byte | Write data length |
|  | btAryBuffer | byte[] | Write data |
| return (int) | success：0；fail：-1； |  |  |
| Reference Code |  |  |  |

## Return value error code table

| Error code | description |
| --- | --- |
| 0 | Execution succeeded. |
| 1 | The electronic tag was not found. |
| 5 | The access password is incorrect. |
| 9 | Destroy password errors. |
| 10 | The destruction password cannot be all zeros. |
| 11 | The command is not supported for electronic tags. |
| 12 | For this command, the access password cannot be 0. |
| 13 | The electronic tag has been set with read protection and cannot be set again. |
| 14 | The electronic tag is not set to read protection and does not need to be unlocked. |
| 16 | Byte space is locked and write failed. |
| 17 | Cannot be locked. |
| 18 | Already locked and cannot be locked again. |
| 19 | The parameter failed to be saved, but the value set was valid before the read-write module was powered off. |
| 20 | Cannot be adjusted. |
| 248 | Antenna detection error |
| 249 | An error occurred in the execution of the command. |
| 250 | There are electronic tags, but the communication is not smooth and it cannot be operated. |
| 251 | No electronic tags are operable. |
| 252 | The electronic tag returns an error code. |
| 253 | The command length is incorrect. |
| 254 | Illegal orders. |
| 255 | Parameter error. |
| 48 | Communication error. |